CPE301 – SPRING 2019

# Design Assignment 1A

Student Name: Itzel Becerril

Student #: 2000478001

Student Email: becerri2@unlv.nevada.edu

Primary Github address: https://github.com/HadidBuilds/hw_sub_da1.git

Directory: DA1A

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Used the assembler in Atmel Studio.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

Registers before assembling.

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

;

; DA1A.asm

;

; Created: 2/15/2019 12:18:20 AM

; Author : itzel

;

; Replace with your application code

.org 0x0000

START:

LDI R25, 0X10   		;High byte multiplicand = 16

LDI R24, 0X01 		;Low byte multiplicand = 1

;high and low together is 4097

LDI R22, 0X11  		 ;multiplier = 17

LDI R26, 0			 ;set to zero for carry

MOV R27, R22		 ;saved iteration for loop

INC R27 			 ;increment by one to compare to breq

CLR R18			 ;resets R18

CLR R19			 ;resets R19

CLR R20			 ;resets R20

CPI R22, 0			 ;compares R22 to 0

BREQ DONE 		 ;if it is equal to 0 then go to done

CONTINUE:

DEC R27 			;dec counter

BREQ DONE 		;if not equal to 0 jump to done

ADD R18, R24		;add

ADC R19,R25 		;add carry

ADC R20, R26 		;add carry

JMP CONTINUE 		;jmp back to continue

DONE:

BREAK 			;answer should have 011011 = 694649

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

REG: R20:R19:R18 = 01:10:11

Execution time at 16MHz/cycles = 9.25us

Verification with high level language:

- GITHUB LINK OF THIS DA

https://github.com/HadidBuilds/hw_sub_da1.git

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Itzel Becerril